| Chapter | 19 |
| --- | --- |
| ESA Space Weather Activities |  |
| Eamonn J. Daly |  |
| Space Environments and Effects Analysis Section, European Space Agency, ESTEC 2200 AG Noordwijk,The Netherlands |  |

Abstract	The European Space Agency (ESA) is undertaking a space weather initiative in which preparatory studies are being performed and developments are being made to pave the way for a possible future ESA space weather programme and a possible European space weather service. This initiative is based on long-standing activities in analysis of space environments and their effects on European space programmes and on a successful solar terrestrial physics programme over many years and many missions. This chapter describes these activities, discusses space weather effects, outlines the goals of the present phase of ESA’s initiative and discusses future directions.

Keywords	ESA, space weather, space environments and effects.

1.	Introduction

The natural space environment represents a considerable hazard for spacecraft and the European Space Agency (ESA) has for many years taken measures to ensure that its spacecraft are able to survive and operate in it. As spacecraft and their payloads have become more sophisticated, so their susceptibilities to effects induced by the environment have increased. Consequently ESA has, like other organizations, increased its efforts in analysing these problems and in developing the means to understand and anticipate the environment and to avoid the effects. This environment and its complex behavior are also subjects of intensive scientific investigation within solar-terrestrial physics. Space weather encompasses a broad range of phenomena (solar, interplanetary, geomagnetic, ionospheric, atmospheric and solid earth) impacting space and terrestrial technologies. It is a subject which is of relevance to developers of technological systems but one which relies on characterization and understanding of the solar-terrestrial system.

A few high-profile space weather events have drawn attention to the effects. For example, the hazards to spacecraft from electrostatic charging (Baker et al, 1996, Fredrickson, 1996, Wrenn, 1995) and to ground-based power networks from induced current surges (Kappenman and Albertson, 1990, Lanzerotti, 1979). But these are just the "tip of the iceberg" of more numerous, less well-publicized problems and implications. There is consequently a growing appreciation that as society becomes more reliant on space-based systems for services such as communications and navigation, the disruptions to these services from space weather has become a serious issue. Apart from disruption to commercial space activities, scientific missions can be seriously affected because of their use of highly advanced technologies. Recent examples which will be further discussed are the effects on the Chandra and XMM (X-Ray Multi-Mirror) Newton X-ray astrophysics missions. The analyses of potential problems on these missions, and continuing evaluation in-flight, are good examples of the needs for accessible space weather resources including databases, "predictive" models and near-real-time data. Furthermore, over the next few years manned spaceflight will undergo a considerable expansion with the exploitation of the international space station. Space weather, in the form of enhancements to energetic particle radiation, is of crucial importance for manned space activities. Space radiation also penetrates the upper atmosphere where crew and electronic systems on aircraft can be affected. Finally, ground-based systems such as power distribution networks, pipelines and ground-to-ground radio communications can also be seriously affected.

At the time of writing, we are near the maximum of the current cycle, cycle 23. The increased chances of solar flares, coronal mass ejections and solar energetic particle events have led to added interest in space weather both from the space community and from the general public. This interest is supported by an array of excellent solar-terrestrial science missions such as the joint ESA-NASA Solar and Heliospheric Observatory SOHO (Huber and Wilson, 2000). However, it is important to recognize that the effects of Space Weather are present throughout the solar cycle and that some important aspects can be more severe away from solar maximum. For example electron flux levels in the Earth's outer radiation belt are generally higher during the decaying phase of the solar cycle.

The space weather discipline draws from both the space environments and effects domain and from the solar-terrestrial physics domain. ESA and European research agencies have strengths in both areas. There is considerable interest in Europe to investigate the marriage of the technological and scientific capabilities to address perceived user needs for space weather products and services. Whereas co-ordinated Space Weather activities are well established in the US, Europe has yet to undertake a co-ordinated program in this area. Past ESA workshops and studies identified the needs as well as possible European approaches to the subject (ESA, 1998, Koskinen et al., 1999). An important step towards a co-ordinated European Space Weather program has recently been taken with the initiation of broadly based studies in the context of the ESA General Studies Programme. Major parallel studies are laying the groundwork for a possible operational European space weather service. These studies will be discussed further later in this chapter.

2.	Space Weather Problems for ESA Programmes

2.1	General

During the development of spacecraft, the expected environment needs to be carefully considered. The development process includes analyses of possible problems from the space environment and the implementation of appropriate measures to avoid or cope with effects of concern. Analyses make use of information on the environment in the form of models and tools which have developed over the years to cope with an evolving set of problems. Some of the effects on space systems are summarized in Table 1.

In this section, the major environmental effects will be outlined and their connection with space weather described.

2.2	Radiation Effects

As in other space agencies, ESA's concerns with space weather effects probably began with concerns over radiation effects on spacecraft systems. This radiation environment is due to sporadic solar particle events, energetic protons and electrons trapped in radiation belts and cosmic rays (e.g. Daly, 1989). The effects of these in damaging solar cells, electronic components and inducing upsets in large-scale-integrated electronics had long been taken into account in spacecraft development. In the last decade there has been a general increase in radiation-related problems and new types of problems have arisen.

While the preoccupation in the 1960's and 1970's was very much with the damage caused over the lifetime of a spacecraft to solar cells and components, in later years a number of other effects have arisen. The changes in states of logic elements in integrated circuit induced by the charge trail left by passage of a single energetic ion, known as single event upsets (SEU), have become a major concern since effects on the spacecraft controlling functions could have devastating consequences. Single event rates can often be coped with when they occur in non-critical memories such as data stores and can often be corrected by special circuitry. Dramatic increases in the SEU rates often occur during solar energetic particle events as shown in Figure 1 from the SOHO mass memory unit. The rate rose by a factor of 6 at the peak of the November 1997 event and by a factor of 300 at the peak of the July 2000 event.

| Environment | Effects |
| --- | --- |
| High Energy Radiation: |  |
| Cosmic Rays | Upsets in electronics; Long-term hazards to crew; Interference with sensors; |
| Solar Energetic Particle Events | Radiation damage of various kinds; Upsets in electronics; Serious prompt hazards to crew; Massive interference with sensors; |
| Radiation Belts | Radiation damage of various kinds; Upsets in space electronics; Hazards to astronauts; Considerable interference with sensors; Electrostatic charging and discharges |
| Near-Earth Plasma Populations: |  |
| Geomagnetic (sub-) storms | Electrostatic charging and discharges; |
| Ionospheric Effects | Communications disruption; Navigation services disruption |
| Others: |  |
| Atmosphere | Increased drag on spacecraft and debris; Attitude perturbation |
| Meteoroids | Spacecraft damage |

Table 1. The various space-weather environments and their effects.

Another source of interference is radiation background. Imaging detectors such as, charge-coupled devices (CCD's) are used in a variety of space applications including as imaging elements in space telescopes, in Earth observation systems or in star trackers of attitude control systems. Particles impacting a detector can give rise to signals which appear as "noise" on the image, sometimes completely overwhelming the image. Examples from the ESA-NASA SOHO spacecraft are shown in Figure 2. These images were taken during the July 2000 solar proton event and show heavy contamination of the image from particle hits on the detector. Such contamination is a feature of many space-borne detectors.

Figure 1. SOHO mass-memory single-event upsets. The spikes in November 1997 and July 2000 are due to solar particle events

ESA's Infrared Space Observatory and Hipparcos spacecraft also experienced background effects and it is also a feature of the XMM-Newton detectors. In many cases it can be removed with image processing software but if heavy contamination is present during space weather events, the data are lost. It is becoming more common to use star trackers as part of the attitude control systems of spacecraft. These help orient the spacecraft by recognizing sets of star patterns. If the image contains a lot of bright features induced by radiation, the system can become confused and several examples are known where this has occurred and led to loss of attitude. Clearly, the image background during solar energetic particle events will be very much higher than normal.

2.3	Electrostatic Surface and Internal Charging

The importance of space weather to space systems increased in the 1980's as a result of several cases of operational anomalies on geostationary communications and meteorology spacecraft. The anomalies were attributed to high-level electrostatic charging of surfaces which led to discharges and electromagnetic-induced disruption of spacecraft systems. The charging events were associated with surges of hot plasma flowing into the parts of the magnetosphere around the geostationary altitude (about 36000km) during geomagnetic "sub-storms". The affected missions include the Marecs marine communications test satellites, the ECS series of communications test satellites and the pre-operational satellites in the Meteosat meteorological satellite series. In investigating the Meteosat-1 anomalies, a decision was taken to put a plasma environment monitor on the second spacecraft in the series. The analyses of these data and their correlations with anomalies led to the conclusion that the anomalies were not due to high level surface charging.

Figure 2. Images taken by the LASCO (left) and EIT (right) telescopes on the joint ESA-NASA SOHO mission during the July 2000 solar energetic particle event showing severe effects on the detector from radiation background.

About this time, it was noted by Baker et al. (1987) that anomalies on US spacecraft correlated with energetic (~MeV) electrons, implying that penetrating electrons could induce charging and discharging within spacecraft by collecting in dielectric materials or ungrounded metallic parts. Since it was a likely source of the Meteosat anomalies, Meteosat-3 contained a detector to monitor these higher energy electrons. The data showed very clear correlations with anomalies (Rodgers et al., 1998). Figure 3 shows a superposed epoch analysis of the >2MeV electron fluxes, measured in this case by a detector on the GOES geostationary satellite, for all anomalies of a particular type. This shows the average environment preceding the anomalies. The clear increase in energetic electron flux is highly indicative of internal charging as a source. Similar behavior was also reported by Wrenn (1995) for a classified UK defense satellite during the 1990's. Furthermore the energetic electron flux before the failures of the primary and back-up processors on the Equator-S mission strongly suggest that internal charging led to this total satellite loss. The environment measured by a detector on GOES-8 is shown in Figure 4 where it can be seen that preceding the failures the energetic electron fluxes were high as a result of injection events ("storms"). Equator-S was in an eccentric equatorial orbit crossing the radiation belts while GOES is in geostationary orbit. Equator-S was therefore probably exposed to a more severe environment than GOES measured. These European examples are in addition to several cases reported in recent years in the US.

Figure 3. The average >2MeV electron environment preceding a particular type of anomaly on Meteosat-4. On average, the flux of energetic electrons increases by orders of magnitude before an anomaly

2.4	The Role of Models

To ensure that spacecraft will operate correctly in the presence of these effects, it is necessary during the development process to use models of the environments and effects for analyses, and to undertake appropriate testing. Models are intended to address the needs of the space system developer and for efficiency and usability reasons often simplify the physics involved in the phenomena. Even when physical understanding or information is incomplete, the threat still needs to be countered with some quantitative method, albeit of limited validity. It is nevertheless a long-term objective for this community to have models available which are both physically accurate and responsive to the users needs. A good example is in the area of radiation environments and effects where for many years developers have used the "standard" AP-8 (Sawyer and Vette, 1976) and AE-8 (Vette, 1991) models of the radiation belts. These models are known to be weak and do not represent the dynamic ("space weather") behavior of the electron belt. Nevertheless, in the absence of anything better, they have continued to be used. Developments have recently given hope but there still remains a usability problem.

Figure 4. The environment in geostationary orbit as measured by GOES-8 detectors for the periods around the Equator-S primary and backup processor failures, indicated by the arrows

ESA's Space Environments and Effects Analysis Section has responsibility for supporting the development of ESA missions. The service it provides includes assessments of elements in Table 1. In parallel with this support function, it is responsible for the initiation and execution of technology R&D as part of a space environments and effects technical domain of ESA's Technology Research Programme (ESA, 1999). This R&D has led to developments of tools and models, as well as R&D for longer-term application. In doing this R&D and support work, the section is closely in touch with the user needs for space weather data for space system applications. Current R&D activities include (ESA, 1999):

- The Space Environment Information System (Spenvis) (Heynderickx, 1998). This is an internet/intranet-based system containing a wide range of models, tools and data concerning many aspects of space environments and their effects on space systems. It is targeted at the space systems developer who needs rapid reliable access to authoritative (often standard) methods. The system also contains link to the European ECSS engineering standard on space environment (ECSS, 2000).

- Modelling of the Earth's radiation belts where various high altitude and low-altitude data sets are studied to validate or improve models of the radiation belts. The activity also includes detailed comparison between a physical model, Salammbô, and spacecraft data of energetic electron belt dynamics

- Development of data-based analysis of space environments ("SEDAT") (CLRC, 2000) where existing spacecraft data sets are interrogated by standard and user-defined methods to derive custom "models".

- Development of engineering tools for assessment of the hazard from charging of materials inside spacecraft by energetic electrons ("internal charging") (Sørensen et al., 1999). This research also investigated the way to specify the hazard for design and the associated test methods.

- Participation with the high-energy physics community in a world-wide effort to produce a next generation of object-oriented tool-kit for simulations of particle interactions with matter ("Monte-Carlo codes"), Geant4 (Apostolakis, 2000). This effort was initiated by CERN, the European center for nuclear research. ESA's activity has resulted in space-specific features for Geant4 (Truscott et al., 2000).

- Developments of space environment monitors and the analysis and exploitation of data from them (Bühler, 1998, Desorgher at al., 1999, Daly et al., 1999).

- Analysis of electrostatic charging behavior of spacecraft in polar orbits and analyses of the correlations between anomalies and environmental parameters (Andersson et al., 1998). These studies also included research on tools for anomaly predictions (Wu et al., 1998).

- Research on AI methods in spacecraft anomaly analysis and prediction - the SAAPS (Spacecraft Anomaly Spacecraft Anomaly Analysis and Prediction System) (Wintoft, 1999).

Many other important activities have been undertaken including activities related to Martian environments, micro-particle impacts and contamination (ESA, 1999).

3.	Space Weather and Space Environment Support to Project Development

As mentioned, a key task is to support space systems development.  Virtually all ESA spacecraft are supported, starting early in the process with the mission concept definition. A good case history is the XMM-Newton X-ray astronomy mission.

X-ray astronomy in space relies on the focussing of X-ray photons by low-angle scattering from shaped "shells". In most cases the "optics" consist of two sets of nested concentric shells with shapes near to sections of cones. Two grazing-incidence scatters result in focussing of the X-rays on the shell axis. ESA's XMM-Newton mission has three mirror modules of outer diameter 70 cm, each consisting of 58 nested shells which focus the X-rays onto CCD detectors some 7 m from the mirrors. XMM is in a highly eccentric orbit of apogee 114000km, perigee 7000km and inclination 39°. In this orbit it is subjected to fluxes of electrons and ions of various energies from magnetospheric and heliospheric sources.

Recently, an intensive investigation was undertaken to study potential problems to detector operation from medium-energy (100's of keV's) protons (Nartallo et al., 2000). CCD detectors are known to be radiation sensitive and much attention is given to shielding them against radiation penetrating spacecraft structural materials. However, it was found that protons of energies in the range of hundreds of keV to a few MeV could scatter at low angles through the mirror shells. These protons, because of their low energy can produce a high non-ionizing dose in unshielded CCDs and therefore pose a potential threat. Historical data on the interplanetary and magnetospheric low-energy proton environments were interrogated to determine the magnitude of the threat. Complex modelling of particle propagation through spacecraft systems was undertaken with the Geant4 Monte-Carlo toolkit. The datasets were used to establish details of observing time expected to be lost in protecting the CCDs from sporadic particle flux enhancements by closing protective shields. Several interesting points regarding space weather can be made as a result of this analysis:

- Crucial data sets used for the analysis of mission-critical engineering problems were produced by science missions (IMP, SOHO, ACE, Equator-S, ISEE) which could never foresee such applications;

- XMM-Newton has an on-board radiation monitor, to which there was resistance early in the project preparation. It is now an important resource on the spacecraft;

- Spacecraft operators are keenly interested in the state of the space weather and would certainly make use of predictions of sporadic particle enhancements should they be available.

All this effort was in addition to several space environment related analyses carried out in the course of the definition and development of the XMM-Newton mission over the preceding 10 years. In such a process, early analyses of the environments of orbit options were undertaken, followed by detailed analyses related to the final orbit and the radiation doses and particle fluxes to be anticipated for electronic components and detectors. Further analyses included assessments of the electrostatic charging hazards, analysis of the potential problems from micro-meteoroids (punctures to telescope tube and hazards to fuel tanks) and detailed analysis of radiation background sources (Dyer et al., 1995, Hilgers et al., 1998). XMM-Newton was launched in December 1999 and is operating well.

4.	ESA's Science Activities

ESA's science program is related to space weather in two ways. Space weather effects on science missions are an increasing concern, while on the other hand science missions can contribute crucially to space weather research. As space science missions become more complex and demanding, the need to design tolerance to space weather effects into scientific payloads as well as spacecraft systems becomes more important. Examples include sensitivity to radiation, leading to increased backgrounds and even detector damage, as well as the complete failure of key components. As mentioned above, these issues were of concern to the recently launched XMM-Newton mission (Nartallo et al., 2000).

An important spin-off of scientific missions can be to show what is possible for future service-oriented ventures. For example, the joint ESA-NASA SOHO mission is a key member of the fleet of spacecraft studying the Sun and its effects on the interplanetary environment. It is also highly useful as a resource for providing Space Weather warnings. ESA's scientific studies related to Space Weather phenomena were further enhanced with the launch of the Cluster II satellites.

As part of the competitive process for selection of future science missions, ESA recently studied future medium-sized missions. Among these were the STORMS and Solar Orbiter proposals, both of which could contribute to the world-wide space weather effort. STORMS was proposed as a set of 3 spacecraft in eccentric near-equatorial earth orbits. With apogee at about 8 Earth radii, the spacecraft pass through the radiation belts and the ring current regions. As the name suggests, the principal motivation for the mission was to study the physics of geomagnetic storms and the inner magnetosphere's responses to them. The spacecraft would carry particle and fields instruments and energetic neutral atom imagers. Solar Orbiter was proposed to orbit the sun as close as 40 solar radii (0.19 AU) and to carry out detailed solar remote sensing. Its orbit would also take it to helio-latitudes of about 33º. For part of the time the orbit would be quasi-co-rotational. Spectroscopy and imaging would be performed at high spatial and temporal resolution, along with in-situ sampling of particles and fields. Both proposals were highly rated and eventually Solar Orbiter selected for implementation. Launch is presently planned for 2009.

5.	Efforts Towards a European Space Weather Programme

Recognizing that there is a growing need for space weather related data for ESA programs, and also that there were issues related to the impact of space weather on non-space technologies which could be important for Europe, ESA took steps to analyze the subject in detail. While not the first ESA activity, a workshop held in 1998 (ESA, 1998) was an important event which brought together the user, science and technology communities to explore the possible ways forward. It was clear that user needs were growing. At the same time, the maturity achieved in solar terrestrial physics, allied to technological advances (in-orbit monitoring, ground-based computing power, etc.) meant that it was certainly feasible to deliver products for users in the short term and contemplate considerable improvements to them over the medium and long terms. These improvements would imply developments in the systems deployed in space and on the ground for space weather monitoring and in the science, simulation, modelling and delivery aspects of the ground-based activities. As a result ESA approved the execution of parallel wide-ranging studies. The two studies (ESA, 2000) are led by Alcatel Space and Rutherford Appleton Laboratory. In each consortium there is a strong blend of technology, science and applications. The top-level goals of the studies are to:

- investigate the needs for and the benefits of an ESA or other European space weather program

- establish the detailed data supply requirements by detailed consideration of the quantification of effects and intermediate tools;

- perform detailed analysis of potential program contents:

a detailed definition of the space-segment

a detailed definition and proto-typing of the service-segment

- perform an analysis of  collaborative and organizational structures which need to be implemented by ESA and member states

- provide inputs and advice for preparation of a program proposal, including project implementation plan, cost estimate and risk analysis.

In association with these activities, ESA has also established a Space Weather Working Team consisting of European experts in various space weather and user domains, to oversee the activities and advise ESA on future activities.

While there is considerable interest in space weather in Europe, initiating any major new ESA space weather activity requires the agreement of national delegations to ESA's decision-making committees. Such a commitment can only be made after the needs for such expansion and the demonstration of its benefits are clearly established. The more scientific aspects will probably be the responsibility of ESA's science program where proposals are subject to the well-established peer review selection process. While technological research and developments will continue into space environments and effects, any large expansion of these activities for ground- and space-based space weather infrastructures is conditional upon high-level approval. The above studies and associated activities are crucial in establishing the justification.

In ESA member states many important activities related to space weather are being undertaken as part of national programs (ESA, 1998). These include activities addressing military needs. The interests of ESA's various member-states also differ. For example, Scandinavian and other nations at high latitude are keenly interested in effects on power systems, pipelines and other ground systems from auroral electrojet induced ground-level currents.

It will also be important to consider how any eventual service will be implemented in Europe. ESA's role is as an initiator and developer of technologies. The provision of fully operational end-user services should be provided by other organization in a way analogous to satellite communications or meteorology.

6.	Conclusions

The wide-ranging activities of ESA in the space weather and space environment domains have been summarized and recent important examples of space weather concerns given. In particular, the space weather effects on XMM and efforts to analyze these effects and other space environmental hazards illustrated the depth and breadth of the work that is typically necessary in this domain when preparing a complex space mission.

We have highlighted the important scientific and technological contributions that ESA in particular and Europe in general have made. We emphasize that while there is considerable interest in Europe in expanding space weather activities toward a fully-fledged program, this will be as a result of clear demonstration of real needs and benefits. These complex issues are being addressed by on-going studies.

7.	Acknowledgements

I am very grateful to Alain Hilgers of ESA/ESTEC for his considerable efforts related to the ESA space weather initiatives described here. The data used in Figure 1 were provided courtesy of Helmut Schweitzer of the SOHO project team. The images on Figure 2 are courtesy of the SOHO/LASCO and SOHO/EIT consortia of the mission. SOHO is a project of international co-operation between ESA and NASA. The data used in Figure 4 were obtained from the NOAA-NGDC SPIDR service.

8.	References

Andersson L., L. Eliasson, P. Wintoft, Prediction of times with increased risk of internal charging on spacecraft, in Proceedings of the ESA Workshop on Space Weather" ESA-WPP-155, November 1998.

Apostolakis J., Geant4 status and results, CHEP-2000 Conference, to appear in proceedings, 2000

Baker, D. N., J. H. Allen, R. D. Belian, J. B. Blake, S. G. Kanekal, B. Klecker, R. P. Lepping, X. Li, R. A. Mewaldt, K. Ogilvie, T. Onsager, G.D. Reeves, G. Rostoker, H.J. Singer, H. E. Spence, N. Turner, An assessment of space environmental conditions during the recent Anik E1 spacecraft operational failure", ISTP Newsletter, Vol. 6, No. 2. June, 1996.  Retrievable from http://www-istp.gsfc.nasa.gov/istp/newsletter.html

Bühler P., Desorgher L., Zehnder A. and Daly, E., Observation of the Radiation-Belts with REM, in: Proceedings of the ESA Workshop on Space Weather, WPP-155, p.333, ESA, 1998.

CLRC Rutherford Appleton Laboratory, SEDAT Project homepage http://www.wdc.rl.ac.uk/sedat/, 2000

Daly E.J., Radiation Environment Evaluation for ESA Projects, in High Energy Radiation Background in Space, eds. A.C Rester & J.I. Trombka AIP Conference Proceedings 186, NewYork 1989 483-499

Daly E.J., P. Bühler and M. Kruglanski, Observations of the outer radiation belt with REM and comparisons with models, IEEE Trans. Nucl. Sci NS-46, 6, December 1999.

Desorgher L, P Bühler, A Zehnder, E. Daly and L Adams, Modelling of the outer electron belt during magnetic storms, Radiation Measurements 30, 5  pp 559-567, 1999

Dyer C. and Truscott P., The analysis of XMM instrument background induced by the radiation environment in the XMM orbit", DERA report DRA/CIS(CIS2)/CR95032/1.0, final report on work performed for ESTEC Contract No 10932/94/NL/RE, December 1995; http://www.estec.esa.nl/wmwww/WMA/XMM/XMM.html

ECSS (European Co-operation on Space Standards), Space Environment, ed. E. Daly, ECSS-E-10-04, ECSS Secretariat, ESTEC, Noordwijk The Netherlands, 2000.

ESA, Proceedings of the Workshop On Space Weather, ESTEC, Noordwijk, The Netherlands, ESA WP-155, 1998

ESA, Space environments and effects, in: Special Issue, Preparing for the Future Vol.  9, No. 3,  December 1999.

ESA Space Weather Studies www Site: http://www.estec.esa.nl/wmwww/spweather/spweathstudies.htm , ESA, 2000.

Fredrickson A.R., Upsets related to spacecraft charging, IEEE Trans. Nucl. Sci. NS-43, 2, 426, 1996.

Heynderickx, D., Quaghebeur, B., Fontaine, B., Glover, A., Carey, W.C. & Daly, E.J., New features of ESA's space environment information system (Spenvis), in: Proc. ESA Workshop on Space Weather, ESTEC, Noordwijk, The Netherlands, ESA WPP-155, pp. 245-248, ESA 1998.

Hilgers A., P. Gondoin, P. Nieminen and H. Evans, Prediction of plasma sheet electron effects on X-ray mirror missions, Proc. ESA Workshop On Space Weather, ESTEC, Noordwijk, The Netherlands, ESA WP-155 p.293, 1998.

Huber M.C.E. and Wilson A., eds., ESA's report to the 33rd COSPAR meeting", pp.29-33, ESA- SP-1241, 2000.

Kappenman J.G. and Albertson V.D., Bracing for the geomagnetic storms", IEEE Spectrum, , p5, March 1990.

Lanzerotti, L. J., Geomagnetic influences on man-made systems, J. Atm. Terr. Phys., 41, 787-796, 1979.

Nartallo R., E. Daly, H.D. Evans, A. Hilgers, P. Nieminen, J. Sørensen, F. Lei, P.R. Truscott, S. Giani, J. Apostolakis, L. Urban and S. Magni, Modelling the Interaction of the Radiation Environment with the XMM-Newton and Chandra X-ray Telescopes and its Effects on the On-board CCD detectors, Proceedings of the Space Radiation Environment Workshop, DERA Space Department, Farnborough, UK, in Press 2000.

Sawyer D.M. and Vette J.I., AP8 trapped proton environment for solar maximum and solar minimum', NSSDC 76-06, 1976.

Sørensen J., D. J. Rodgers, K. A. Ryden, P.M. Latham, G. L. Wrenn, L. Levy, G. Panabiere, ESA's tools for internal charging., in: Proceedings of the Conference on Radiation and its Effects on Components and Systems (RADECS), 1999.

Truscott P., F. Lei, C. Ferguson, R. Gurriaran, P. Nieminen, E. Daly, J. Apostolakis, S. Giani, M.G. Pia, L. Urban, M. Maire, Development of a spacecraft radiation shielding and effects toolkit based on Geant4, CHEP-2000 Proc., 2000.

Vette J.I. The AE-8 trapped electron model, NSSDC/WDC-A-R&S 91-24, 1991

Wintoft P., Development of AI methods in spacecraft anomaly predictions, IRF Web site http://eos.irfl.lu.se/saaps/ 1998.

Wrenn G.L. Conclusive evidence for internal dielectric charging anomalies on geosynchronous communications spacecraft, J. Spacecraft and Rockets 32, pp 514-520, May-June 1995

Wu, J.G, Lundstedt, H., Eliasson, L., Hilgers, A., Analysis and real-time prediction of environmentally induced spacecraft anomalies, in Proceedings of the ESA Workshop on Space Weather ESA-WPP-155, November 1999.